Ministry of Health of Ukraine

Odessa National Medical University

International Faculty

Department    of Organization and Economics of Pharmacy

with post-diploma specialization

Discipline       "The labor protection in industry"

Year of study   4         Term 8

GUIDELINES FOR PRACTICAL CLASS

Topic  №10 “Electrical safety”

Discussed and approved

at the meeting of the

Department of Organization and

Economics of Pharmacy

Minutes №1 “04/09 2023

Head of the Department,

As. Prof. Oksana BIELIAIEVA

2023

4. The content of the topic.

The responsibility for the organization of safe operation of electrical installations by the Rules rests with the employer, who must: appoint a person responsible for the proper condition and safe operation of electrical installations; to create and staff an electrical service from among persons who have reached 18 years of age, have the appropriate education and passed a medical examination and have no contraindications; develop and approve job descriptions for employees and instructions for safe work performance; to provide training and testing of knowledge of employees, timely inspection of electrical installations and conducting preventive, anti-accident and acceptance tests. A person responsible for the electrical system shall be appointed to directly perform the functions related to the organization of operation of electrical installations. Personnel operating electrical installations are divided into electrical, electrical and non-electrical. Electrical personnel consists of: administrative and technical - managers and specialists who ensure the organization of work; operational, which carries out operational management and maintenance of electrical installations (inspection, preparation of jobs, admission and supervision of workers, etc.); repair - provides maintenance, repair, installation, adjustment and testing of electrical installations; operational and repair - trained personnel for operational maintenance of electrical installations. Electrotechnological personnel serve: electrotechnological installations (electric welding, electrolysis, electrothermia, etc.); installations, the operation of which requires constant adjustment of electrical equipment and electric drives; manual, portable or mobile electrical receivers. Non-electrical personnel are personnel who service production equipment powered by electricity (machines, power tools, other electrified units), as well as persons who may have contact with such equipment (locksmiths, cleaners, etc.). The list of positions and professions of employees of electrical and electrotechnological personnel who need to have an appropriate group for electrical safety, approves the head of the enterprise.

The effect of electric current on the human body.

It should be noted that a significant potential danger of electric shock is the inability of human senses to detect the presence of electrical voltage at a distance. Passing through the human body, electric current has a thermal, electrolytic, mechanical and biological effect, in particular: thermal - causes burns to certain parts of the body, heating of blood vessels, heart, other organs through which the current passes and the occurrence of functional disorders; electrolytic action - the decomposition of blood and other organic fluids, which causes significant violations of their physical and chemical composition; the mechanical action of the current causes such damage as ruptures, stratification of body tissues due to the electrodynamic effect; The biological action of the current leads to dangerous excitation of cells and tissues of the body, which is accompanied by involuntary convulsive muscle contraction and can cause significant disturbances in the respiratory and circulatory system until their complete cessation. The current can pass directly through these tissues or cause a reflex effect on the organs through the central nervous system.

Means of protection against electric shock.

The rules of operation of electrical protective equipment stipulate that in order to create safe working conditions for employees who service electrical installations, it is necessary to provide them with the necessary means of protection. The owner of the funds is responsible for fulfilling this requirement, as well as the requirements for their acquisition, proper storage, timely periodic inspections and tests and the organization of their accounting. Employees who have received protective equipment for personal use are responsible for their proper operation and storage, as well as for the timely decommissioning in the event of a malfunction. Insulating electrical protective equipment is divided into basic and additional. In electrical installations with a voltage of up to 1000 V, the main electrical protective equipment includes: insulating rods; insulating pincers; electric measuring tongs; voltage indicators; dielectric gloves; tool with insulating coating. Additional means of electrical protection include: dielectric footwear; dielectric carpets; insulating stands; insulating pads; insulating caps; voltage alarms; protective fences (shields, screens); portable grounding; posters and safety signs; other means of protection. Additional electrical protective equipment is used together with the main ones. In addition to the above means of protection, personal protective equipment must be used, in particular: protective helmets - to protect the head; goggles and shields - to protect eyes and face; gas masks and respirators - to protect the respiratory system; gloves - for hand protection; seat belts and safety ropes.

PRACTICAL CLASS ELECTRICITY HAZARDS

Task 1. Study the information on electrical safety. Explain the points made in the questions.

https://www.isastur.com/external/seguridad/data/en/1/1_5_5.htm

- Electricity Hazards

- Factors that Intervene

- Effects of Electric Current

- Prevention and Protection

Task 2. Analyze self-inspection checklists for employers to identify electrical safety hazards in the workplace. Present the list of employer actions in the form of a table.Use this source https://www.osha.gov/sites/default/files/publications/small-business.pdf

1https://www.youtube.com/watch?v=OEtsLDZTDcA

Electrical Hazards and Emergency Situations 1/53

2https://www.youtube.com/watch?v=vhtQGQbuq6w

Electrical Work Safety Awareness Training | Electrical safety video animation

3https://www.youtube.com/watch?v=_ZuFJkWT0ZA

Electrical Safety 101 2.41

4.https://www.youtube.com/watch?v=Ybp8sEKOmm0

Electrical Safety 102

https://www.youtube.com/watch?v=oDBqZOOIFno

Electrical hazards and safety

4 https://www.youtube.com/watch?v=1_UciWRyIBM

Safety Sadie Video: Hidden Electrical Dangers Around the Home

5. Information required for the formation of knowledge-skills, which can be found in textbooks:

1.	Unguryan L. M., Bieliaieva A. I., Vishnitskaya I. V., Aleksandrova A. A. Texts of lectures on the discipline "Labor protection in the industry." Odessa: ONMedU, 2020. 111 p.

2.	Unguryan L. M., Bieliaieva A. I., Aleksandrova A. A., Vishnitskaya I. V. et al. Labor protection in the industry: workshop. Odessa: ONMedU, 2019. 48 p.

3.	Unguryan LM, Bieliaieva AI, Vishnitskaya IV, Aleksandrova AA and others. Labor protection in the industry: teaching aid. Odessa: ONMedU, 2019. 60 p.

4.	Voinalovych O. V., Golopura S. M. Occupational Safety and Health. Practical. Kyiv: Center of Educational Literature, 2018. 448 p.

5.	Pedro M. Arezes, João S. Baptista, Mónica P. Barroso. Occupational and Environmental Safety and Health. Springer International Publishing, 2019. 765 p.

6.	Thomas P Fuller. Global Occupational Safety and Health Management Handbook. CRC Press, 2019. 359 p.

7.	Code of Civil Protection of Ukraine No. 5403-VI dated 02.10.2012 (As amended)

Resolution of the Cabinet of Ministers of Ukraine "On the procedure for certification of workplaces

8.	for working conditions" No. 442 dated 1.08.92r. (With changes)

9.	Resolution of the Cabinet of Ministers of Ukraine No. 431 of 23.06.94r. "On the procedure for conducting state examination (verification) of technological, design, technical documentation for the manufacture of means of production for compliance with their regulations on labor protection"

10.	CMU No. 843 dated 10.09.2001r. "On the approval of the criterion by which the degree of risk from the implementation of activities in the field of public health protection is assessed and the determination of the frequency of implementation of planned measures of state supervision (control)."

11.	Resolution of the Cabinet of Ministers of Ukraine "On approval of the licensing conditions for the implementation of economic activities for the production of medicines, wholesale and retail trade in medicines, import of medicines (except for active pharmaceutical ingredients)" No. 929 dated November 30, 2016

6. Questions for self-control

1. Electrotechnological installations

2 . Electrical engineering - basic concepts.

3. Safe work performance

4 . Timely periodic inspections and tests

5. Additional means of electrical protection